Lab 2 Report

Jonathan Monreal (jem177) and Jiawei Wu (jxw585)

Group 8

Design Questions

- What is the control signal frequency specification of the motor driver used on the daughter board?

Typ. 30KHz, Max. 100KHz

- What is the chosen output frequency of your PWM generator?

It is ~100kHz

- Assuming a maximum speed of 6000 rpm, what is the maximum motor speed your design will calculate with the provided 192 count/turn encoder attached to the motor?

The maximum speed our design will calculate is ~3200 rpm given our sampling rate of our measuring speed feedback module.

- How frequently does your design determine the motor's rate of rotation?

The reset frequency is ~12KHz.

- Calculate approximately how long it takes to change the speed goal from 0 to full speed in one direction if the button is held constantly.

It takes about 42 seconds for it to saturate at a maximum speed from zero.

- At maximum gain, what is the pulse width when the goal is minimum and maximum for the free-running, enabled motor?

When the goal is at maximum, the pulse width of the pwm one the loaded direction is measured to be ~90%, which is ~9 µs

When the goal is minimum, the pulse width of the pwm would be at 50% of duty load, which is ~5 µs

Counters

goal-counter

The goal counter is a counter that is associated with the speed up buttons of two directions. It counts an absolute value of 8 bit binary continuously from pressing down the buttons. The direction is handled using a dedicated module.

speed_counter

The speed counter is the counter of measuring the feedback speed read from the encoder, which counts up every time the motor passes a groove, and it gets reset at a clock frequency, ensuring a measure of speed. The measurement is used in the equation of r = k(g-m); feedback speed is also mapped to the LEDs on board.

Code Organization

We followed the code organization so that the codes satisfy the requirements.

For the requirement of gain k (originally 8 bit) to be within 0-1, we used r=k(g-m) directly to generate a 8+8 bit number, then bit shifted and use the first 8 bit for the output, effectively converting coefficient k to be a number between 0-1.

Design Diagram and Module Functionality

Module clock_divider is a clock of low frequency that is associated with the buttons press trigger rate.

Module goal_counter is the counter for goal speed from the button press, it generates a direction that is handled separately.

Module func is the implementation of the equation r=k(g-m) that we would use.

Module pwm determines how much duty cycle the motor would receive.

Module direction_handler takes the direction bit from goal and determines which way the motor turns.

Module speedcount_pwm is a pwm adjusting the reset/count percentage of the clock used to measure feedback speed\

Module speed_counter is the motor speed feedback measurement

Oscilloscope Captures

Counterclockwise max:

Counterclockwise loaded:

Equilibrium:

Clockwise max:

Collaboration

We write draft codes and using github to exchange, working in lab together to debug.